Frequency-switch for the

HPR137 RECEIVER

THE  ZetF

Designed by:				Harrie van Deursen

Date:						August 1998

Translation by:				Wil Pfeifer

Date:						March 2000

Introduction.

Because I found it rather awkward to use a PC to switch the channels on the HPR137 receiver, I decided there must be a simpler way of doing that.

And so ZetF was born.

Basically the circuit consists of an EPROM that holds the I2C code needed to switch the receiver.

A 4060 counter addresses the EPROM to readout the code.

To make construction easy a PC-board measuring 41 x 73 mm was designed that is mounted by means of the 6-position switch on the board.

There are 6 possible frequencies of whom one is reserved for PDUS.

Those frequencies are : 1691 – 1694.5 – 1696 – 1702.5 and 1707.

It has been assumed that you use channel 2 of the LNC1700 (or equivalent ) that subtracts 1557 MHz of the receiving frequency, so the receiver needs to be tuned to respectively : 134 – 137.5 – 141 – 145.5 – 150 MHz.

Because there are two versions of the Philips 816 tuner, the UV816 and the FQ816 there is a possibility to choose between both versions.

# Construction.

The PC-board is single-sided and the switch is used to mount the unit to the receiver front.

In principle only four connections to the receiver are needed :

Ground, +5V ,SCL and SDA.

There is also a 5V output available that can be used to switch the receiver to PDUS by means of a relay if you wish.

# Operation.

In the EPROM 2 x 97 codes per frequency are stored as Clock and Data.

It is therefore sufficient to use address lines A0 – A6 to output the codes.

This commences after resetting the 4060. This reset occurs as soon as the selector switch  is set to a different position, using the fact that during switching there is a short moment that the contacts are open and MR of the 4060 gets temporarily high.

Power-on-reset occurs trough combination R3/C3 so the receiver automatically switches to the chosen position.

As MR goes low, the 4060 generates address-codes 0 – 96 and is stopped by Q2 of the EPROM going high.

Other frequencies reside in a different part of the EPROM and are selected by making one of the address lines A7 – A11 high. A12 selects a databank for the tuner UV816.

If the FQ816 is used that address-line should be low, and you have to cut the connection between pin 1 and 2 of the EPROM.

A13 is kept low, so there is room for another 6 frequencies by making A13 high if ever the need arises .

The diodes together with R10 and R11 for a sort of  “bus” construction so the PC-interface can stay connected if you like.

Frequencies in the original EPROM are:

Position 1=1691   2=1694.5   3=1698    4=1702.5    5=1707    6=1694.5 (PDUS).

New EPROM-files will be made available if the need arises.

# Components.

SW1 is a 2MC 6 position switch e.g. LORIN-UK diam. 26 mm 6 mm shaft.

Make sure that this is a “break before make” type, luckily the most common type.

U1 = 27C128 and eventually a 27C64 can be used (missing only the future expansion possibility).

U2 = HEF4060

R.. =  9-pin Sil resistor  8 x 10k, but any value up to 50k will do.

R1=47k  R2=22k  R3,R11,R12=18k  R9=100k.

C1=1nF  C2=100nF ceramic.  C3=22uF/10Volt tantalum.

D1, D11, D12 = 1N4148 or equivalent.