HRPT RX display schematic.

Input pulses from J3/5, PIC port B7, are fed to IC1a pin 2 and IC4 pin 15. The rising edge triggers IC1a, setting its pin 4 low. As long as pulses are received at pin 2 then IC1a stays triggered.

When the channel pulses end IC1a times out and its pin 4 goes high.

The rising edge is fed to IC1b pin 10 and sets its pin 12 low for time period RC.  This low going pulse is used to latch the channel data into IC3 via pin 5.

When IC1b times out the rising edge on pin 12 is used to trigger IC2 pin 6 high. This high going pulse is used to reset the counter IC4 via pin 9.

The pulses fed to IC4 pin 15 are counted and output in 4 bit parallel mode to the latch display driver IC3.

The circuit utilises the re-triggerable properties of IC1a. Providing the input pulses are continuous and of a suitable period then IC1a stays triggered.