A RECEIVER

FOR HRPT AND PDUS RECEPTION

THE  HPR137

Designed by:				Jaap van Rotten

PCB layout and description: 		Arne van Belle

Date:							July 1997

Revision:					December 1999

Revision 2:					    October 2000

A receiver for HRPT and PDUS reception:

## Introduction

This receiver is suitable for the reception of digital signals on 1698 and 1707 MHz, such as those transmitted by the NOAA polar orbiting satellites.  The acronym HRPT stands for High Resolution Picture Transmission.  The receiver can also be switched, to receive digital signals i.e. the PDUS (Primary Data User Station) signals transmitted from the Meteosat type satellites.  These are the A.. B..  I..  J..  W.. and E.. transmissions on channel 2 (1694.5 MHz).  Unfortunately the A.. and B..  images are partially encrypted and they can only be decoded by means of a “decryption” unit and key.

Chinese HRPT (CHRPT) as is transmitted by Feng Yung 1C can also be received with the same settings as for HRPT.

For a description of HRPT please refer to DE KUNSTMAAN of April 1993.  Another good article was published in the RIG journal of September 1996.

It is possible to construct the receiver to receive either only HRPT or only PDUS.

In order to use this receiver, the following are required:

HRPT:	Parabolic dish 0.9 - 1.5 m or a helical aerial, it is essential that the aerial can be rotated in all planes i.e. azimuth and elevation, to permit tracking of the satellite (either by hand or automatically).

For the weaker and wider Feng Yung 1C signal a minimum size of 1 m is advisable.

PDUS:	Parabolic dish at least 1.20 m (1.50m is better) fixed mounted in the direction of Meteosat (approx. 185o azimuth and 29o elevation in the Netherlands).

In all cases a good low noise amplifier (LNA), mounted directly to the feed is required (refer to the RIG SHF LNA or KUNSTMAAN December 1995).  There should not be any terrain obstructions, such as trees or buildings in the reception path.

Additionally a properly operating Meteosat downconverter is required, which may be either a single or dual channel type (i.e. LNC 1700).

The digital signal is decoded and an image produced by means of a PC decoder card or interface, together with the appropriate software.

For HRPT the following decoders are suitable: The decoder described by Rob Alblas, the TimeStep HRPT card, the Orbit card (Arno Hausmann/SSB, previously Hansen) or the NOAA95 decoder (by Roberto Ferrari & Enrico Falconelli from Italy).

PDUS can be decoded with a TimeStep PDUS decoder card or with the Orbit card.

Depending on the decoder and software used, the minimum required hardware is a 386 PC with a good VGA card and monitor,  as well as at least 70 Mb free Hard Disk space.

By means of a small interface on the parallel port of the PC,  the receiving frequency needs to be programmed once only,  after first switching on the receiver.  This frequency is now permanently stored.  It is intended to develop a controller PCB to facilitate tuning.

Requirements for building the receiver

Some home construction experience is essential and soldering must be precise.  Certain components have to be soldered on both sides of the PCB.  The tinplate box in which the receiver is housed is available for purchase.  Holes have to be drilled and two small tinplate pieces have to be cut to serve as RF shields.  The chokes and the two coupling inductors have to be hand-wound.

The receiver has been designed in such a way that only a limited number of alignment stages are required.  For alignment,  an oscilloscope, voltmeter and preferably digital frequency counter is needed.  If a working Meteosat APT system is already available,  alignment and commissioning will be easier.

Principle of the receiver

The receiving frequency is adjustable from 1691 to 1707 MHz in 62.5 KHz steps. The receiver is provided with Automatic Frequency Control in order to correct the Doppler shift of the NOAA satellites and any drift in the Meteosat converter (after warming up the crystal controlled converter will drift slightly).

The block diagram (refer fig. 1) shows the individual components.

1]	Down converter: 1691 - 1707 MHz to 134 - 150 MHz IF

It is difficult to use a receiver operating directly at e.g. 1707 MHz.  Expensive low loss coax cable is required and the LNA used must produce a considerable amount of gain ( more than 30 dB).  There are plenty of  well performing Meteosat converters around and so are designs for home brew units.  Parts can be sourced easily and this is the reason why a standard down converter has been used in this design.

Both single and dual channel converters are available,  however a dual channel type (covering 1691 and 1694.5 MHz) type is to be preferred.

Depending on the LNA and coax cable used,  10 to 30 meter cable can be used between feed and down converter.  This dispenses with the need to mount the converter directly to the feed in a waterproof container.

There is no need to modify the converter and Meteosat APT reception is still possible.

2]	Front end + mixer to the second IF from 36.7 to 38.9 MHz.  (refer also fig. 2)

By using a UV-816 synthesizer controlled tuner for the 1st IF (134 - 150 MHz),  only one standard crystal is required in the converter for channel 2 (1694.5 Mhz),  or if required channel 1 (1691 MHz).

The tuner is a ready to use aligned module,  as found in Philips,  Sony and other TV receivers.  This tuner has a extensive frequency coverage from 46 to 860 Mhz.,  spread over three bands.  Three “follower” tuned stages are used giving the tuner adequate protection against strong interfering adjacent signals.

The tuner is not as sensitive as compared to a communications receiver,  however the Meteosat converter delivers sufficient output signal.

Synthesizer adjustment and band selection is a little more complicated,  but this can be accomplished by using the parallel port of a PC and a one time adjustment only,  when the receiver is turned on.

The tuner is set using the I2C protocol,  which is a standard protocol used to control a number of components in equipment by means of a serial (two wire) connection.  It is frequently used in TV,  SAT  and audio equipment.

3]		The 2nd IF filter,  amplifiers and AGC

The efficacy of an HRPT or PDUS receiver is determined to a large extent by the quality of the IF filter.

The IF filter must have a theoretical band-pass equal to 4 times the digital data speed.  In the case of HRPT this is 4x 666 KHz = 2.66 MHz.

For PDUS the value is 4x166 KHz = 0.664 MHz.

If the filter is too narrow,  the signal will be distorted,  whilst a filter which is too wide will result in a poor signal/noise ratio,   increasing the chance of interference from strong adjacent RF frequencies.

Often 10.7 MHz is used as the IF because of simplicity,  however this is less suitable for HRPT,  as it is rather difficult to obtain the required band-pass at this low frequency (the band-width equals 25% of the center frequency).

For this reason it was decided to use a SAW (Surface Acoustic Wave) filter as used in all TV receivers produced after approx. 1982.  These filters intended for processing European TV signals are easily obtainable and inexpensive.

SAW filters are fixed tuned at small tolerances.  In principle any SAW filter produced for use in European TV reception (PAL) is suitable.

Unfortunately the IF used in a TV receiver is about 4.5 MHz wide (-6 dB),  actually too wide for HRPT.  On the other hand the SAW filter possesses good characteristics (flat admission and steep side lobes),  which makes it very suitable for HRPT.  The center frequency of the filter is around 36.7 MHz.

In the case of PDUS the SAW filter used to separate the sound from the video carrier is used.  This filter has a center frequency of around 38.9 MHz and a admission of about 1.2 MHz.

By means of two tuned stages,  the desired PDUS band-width of approx. 0.7 MHz is obtained.

The OFWG-3250 filter used contains both the video (for HRPT) as well as the audio (for PDUS) filter in a single package.

The output of the SAW filter is amplified by employing 2 MC 1350 IC’s in an adjustable symmetrical in/out configuration.

It is important that the signal remains at constant amplitude,  regardless of signal strength,  in the following demodulator.

Prior to the demodulator,  the signal strength is measured and the amplification level of the MC 1350’s is continuously adjusted (Automatic Gain Control).

This measured signal is simultaneously used to provide a general signal level indication.  The GAIN control must be adjusted in such a way that this meter will allow for easy alignment of the HRPT or PDUS antenna.

A relay is used to switch from HRPT to PDUS.  Amplifiers,  which are not required following switch over will be turned off.

4]		Demodulating the signal

The demodulator consists of a discrete PLL,  a Voltage Controlled Oscillator (VCO) with mixer and a “loop amplifier”,  which locks the oscillator to the incoming signal.  The mixer uses either the SBL-1 double balanced mixer from MiniCircuits or the SCM-1 which is the smd version.

The VCO is constructed around a special IC,  the MC 1648.  This IC contains a stable,  constant output oscillator with sufficient amplitude to drive the mixer directly.

The loop filter is switchable to the different data speeds of HRPT and PDUS.  The VCO frequency is switched between 36.7 MHz for HRPT  and to 38.9 MHz for PDUS by means of a varicap diode.

The center frequency of the VCO may be fine tuned by means of the TUNE pot,  which allows the demodulator to quickly achieve “lock”,  even with weak signals.

The TUNE pot can also be used to adjust possible minor deviations in the crystal frequency.

In the case of PDUS,  it is sufficient to perform one initial “tuning” operation,  after which the tuning knob position is noted.

For HRPT reception on the other hand there will be a Doppler shift (frequency shift during the course of the satellite pass) of +/- 40 KHz at 1707 MHz.

If adequate signal strength is available (depending on the aerial and LNA),  it should be sufficient to adjust the tuning knob once only,  at the time the satellite appears over the horizon,  the PLL system will continuously adjust the receiving frequency (AFC).

If the HRPT signal is rather weak,  it is possible to “tune along” by adjusting the tune control a few times during the NOAA pass.

The tuning meter is very helpful to exactly tune the signal.

It is also possible to check tuning by connecting an oscilloscope to the “eye” connector.

5]		Further processing of the demodulated signal

The analogue data signal,  often also referred to as the “eye pattern” (because on the oscilloscope it appears similar to two eyes),  is amplified and buffered,  allowing it to be sent to the TimeStep decoder card via shielded cable.

At the same time the eye pattern is fed to a comparator via a low pass filter,  which converts the signal to a TTL compatible (5 V) data signal.  A 7404 IC inverter buffers the signal and simultaneously provides an inverted signal.  The non inverted TTL signal may be connected to the Rob Alblas  HRPT decoder.

Please note that the comparator section is not intended for maximum performance. To get the best out of the receiver I recommend using the analogue Eye output and an analog decoder that uses clock-recovery and an integrator.

Construction of the receiver (refer fig. 3)

The PCB measures 160 x 100 mm and is provided with an earth plane on the top.  Areas around connections which are not required to be earthed have been left free and the PCB fits in a standard “Euro box”.  The VCO is shielded on both sides by means of  shaped tin-plate sheets.  The tuner pins protrude through the side of the box and is soldered in place.  Connections required for the power supply wires are provided with feed-through capacitors,  soldered into the box.

An optional output for the 38.9 MHz IF signal may be incorporated for use in subsequent developments of the receiver such as APT and/or LRPT reception.

Winding the inductors

Coils L5, L7, L9, L10, L11, L13 and L14 are chokes which can be prepared by winding 5 turns of  0.3 mm enameled copper wire around a ferrite bead of 3 mm length and a hole of at least 1.2 mm.

L8 and L12 are torroid transformers,  wound on a torroid of approx. 6 mm diameter,  refer attached drawing in fig. 4.

The primary winding consists of 10 turns center tapped,  use 0.3 mm enameled copper wire and ensure that the two windings of 5 turns are wound adjacent to each other in the same direction.

The secondary winding consists of 3 turns wound in the same direction as the 5+5 windings,  but on the opposite side of these.

To allow further development of the receiver later on,  a second secondary winding,  the same as the first one  is provided on L8.

Make sure the correct ferrite material FT 23-61 (=4C65) Violet with u=120 is used.

When mounting L8 and L12 on the PCB,  ensure that there is a 3 mm minimum clearance under the torroid to prevent excessive damping of the transformer through the copper surface.

Modification of  L3,  the VCO inductor:

L3 is a standard TOKO 10 mm coil type KXCSK3267,  which however is fitted with an internal 22 pF capacitor,  which is too large for the VCO circuit.

For this reason the internal C (only for L3 !) has to be removed.  Place the coil upside down with the pins facing you,  on the bottom,  a small silver/brown colored tube should be visible (in between the core and the earth lug of the case).  Press a small screwdriver in the middle of this ceramic tube until it breaks and pulverize what remains until only the soldered ends remain.  Blow away any grit and put a mark on the coil housing with a felt pen to avoid mixing the coil up with L4 and L6 during receiver construction.

Preparing the receiver PCB

Drill all holes in the PCB with a 0.7 - 0.85 mm drill,  the octagonal pads are drilled with a 1.0 mm drill,  as these are connection or test points.  The holes for the trim-pots,  trimmers and coil covers should be drilled with a 1.1 - 1.3 mm drill.  Ensure components fit correctly before commencing soldering.

If the SMD  SCM1 diode mixer is used,  there is no need to drill the holes marked under “SBL1/SCM1”.

Any rough edges on the PCB should be smoothed out with a file,  also ensure that the PCB fits into the tin-plate box properly by filing rebates in the PCB to allow for the overlap at the box corners.

Use a 3 - 3.5 mm drill to drill all holes in the sides of the box according to the sketch provided (fig. 5).  In particular ensure that the holes for the UV816 tuner are in exactly the right position.  The slots for securing the tuner may be drilled with a 1.5 mm drill,  followed by filing to a slotted hole in the box sidewall.  When the tuner fits properly in the tin-plate box,  the position of the slots should be marked on the PCB, so that matching slots can be filed in the PCB.

Order of construction

First solder the jumper wires J1 to J5 and all flat mounted components.  All pins which have not been insulated on the earth side must be soldered on both sides of the PCB !

Do not yet mount any high profile components near the PCB edges,  such as C37, C54, L4, L6, C5, C36.  Also do not at this stage mount the mixer and P4,  because the VCO has to be tested first,  followed by soldering the shield in place.

Do not use an IC socket for U1 and solder the MC 1648 as close as possible to the PCB.  Solder pins 7 and 8 to both the ground and component side.

Fit the PCB in the tin-plate box,  check the correct height of the PCB in the box (refer fig. 5) and solder the PCB all round to the box (solder completely all round on the component side and on the earth side in those places where the ground plane touches the sides of the box !).

Solder the feed-through capacitors for feeding all power supply connections to the VCO compartment and in place.  Fig. 3,  the connection diagram,  shows in which holes a feed through capacitor should be soldered.

Make sure that feed-through capacitors are not used for signal input and output connections and that these wires are fed through the holes in the box.

Now fit the remaining components of the VCO (but not yet any other remaining components).

Testing of the VCO

Connect +5 V to the VCO and check that the oscillator works,  current consumption should be around 22 mA (+5 V, VCO only).  Now check that the VCO oscillates at the correct frequency by connecting a frequency counter to the junction of L2/C11.  The VCO should oscillate at around 34 - 36 MHz.  Alternatively an FM radio could be used,  to receive the third harmonic.  Adjust the radio to 3 X 36 = 108 MHz and position the aerial approx.  10 cm above the VCO,  whilst slowly turning the L3 tuning slug.

When the VCO operates correctly,  construction may continue.

If the SMD double balanced mixer is used (SCM1),  the device has to be soldered to the bottom (solder side),  as close as possible to the PCB.  If the pin device is used (SBL-1) placement has to wait,  refer later comments.

Pin 1 of the SCM1 mixer is identified on top through two white dots.  Pin 1 of the SCM1 must be connected to the earth PCB pad which is connected to pin 6 of relay RE 1.

Please note:

Because the SCM1 is soldered to the bottom side of the PCB,  the pin numbers do not correspond with the detail shown in the component overlay.  Pin 1 becomes pin 2,  pin 3 will be pin 4,  pin 5 will be pin 6 and pin 8 is used in place of pin 7.   Because the double balanced mixer is completely symmetrical this will not influence proper operation.

Prepare the shield panels as shown in fig. 5 (use tin plate or brass of approx. 0.3 to 1.0 mm thickness) and solder these to the PCB on both sides.,  as shown in the print lay-out drawing (fig. 3).  Do not forget to file rebates in the panel used on the solder side.

The other components may now be put in place and soldered.

Do not use IC sockets for the surface filter,  U7, U8, U9, U10, RE1 and the SBL-1 mixer,  but solder these components directly to the PCB (36 - 39 MHz signals are involved here).

Mounting the SBL-1 double balanced mixer

The pins of the SBL-1 are pushed through the PCB and soldered only on the solder side.  Pin 1 of the SBL-1 is identified by a different color of the insulator between cover and pin (blue instead of green).

Remaining holes which have not been insulated from earth on the component side must be connected through to the solder side by means of a piece of wire (through-metalisation).  For this purpose resistor cut offs may be used.

The last component to be mounted is the UV816 tuner,  the pins of which are pushed through the tin-plate box and soldered level on the PCB on the solder side.  Use a heavy duty soldering iron to solder the two tabs on the tuner to the inside of the box and the PCB.

Check all solder joints and ascertain that solder blobs do not cause undesired shorts.

Fig. 5 shows the positions in which holes are to be drilled in the top lid of the box to facilitate alignment.  Cut out a copy of the drilling template and place this on the inside of the lid.

Connection of the external components,  refer fig. 3

Wire the box,  switches,  pot meters,  etc. according to the connection diagram in fig. 3.

Solder 2 pole mini-module plugs to the supply wires,  which match the ones on the power supply.

To connect the tuner control signal,  refer to the “construction of simple PC to I2C interface” below.

If the Meteosat converter is to be powered via the coax cable,  components C59, L15 and C62 are to be mounted on a right angled antenna plug (male),  as shown in fig. 9.  The input of the UV816 tuner is connected to earth by galvanic means.

Checking the voltages and currents

Connect the voltages to the receiver one - by - one.  Push the power supply pin only on the +ve pin of the power supply and using an amp meter make momentary contact with the earth connection of the power supply and earth on the plug.  Ascertain that the observed current consumption corresponds with the values shown below.  Once this has been confirmed,  connect the plug properly with the + and - poles and check voltages as follows:

Voltage		Current consumption

+ 5V			approx. 76 mA

+ 12V			approx. 160/170 mA (PDUS/HRPT)

- 12V			approx. 13 mA

+ 33V			approx. 6 mA

Measurements refer to the complete receiver in the tin-plate box,  however excluding the Meteosat converter !

The LNC 1700 converter  consumes approx. 90 mA at a supply voltage of 12 V (without heater !)

A 1700 MHz LNA should consume a maximum of 65 mA at +12V (3 stage RIG-LNA).

If considerable variations in voltages and current consumption are noted,  check that all wiring has been completed as per fig. 9 and that all IC’s have been positioned correctly.

Check for a clean DC supply on all outputs using a scope or AC-voltmeter.

A power supply for the receiver (fig. 6)

The power supply provides voltages required by the receiver and additionally can be used to supply power for the Meteosat converter and the LNA.

By using a voltage doubler,  the 33V tuning voltage is derived from the same transformer.  The power supply is built on a PCB which is mounted on the side or on the back side of  the case housing the receiver.  The case also serves as heat sink.

Building the power supply,  refer fig. 7

Drill all of the large pads (transformer, electrolytic condensers, connector blocks,  using a 1 mm drill,  use a 0.7 - 0.85 mm drill for the rest.  Use a 3 mm drill for the following holes:  the hole beneath F1,  the holes at the corners of the transformer,  the hole between L1 and L2.  Use a 6 mm drill for the four holes near the voltage regulators VR1 to VR4.

If a PCB mount transformer is not used,  a 3 pole connector block may be mounted on the PCB and the PCB can be trimmed off with a saw next to the curved track.  This eliminates the primary fuse on the PCB and this will have to be mounted on to the case.

Bend the outer legs of each voltage regulator slightly outwards,  followed by bending them up at right angles, just below the thicker part of the legs.  The legs are pushed through the holes from the solder side of the PCB to allow the regulator to lie flat with the markings facing the PCB (on the solder side).

Two jumpers will have to be connected with some insulated wire on the component side.

The completed power supply PCB is mounted either at the back of the receiver case or at the side.  When doing this the voltage regulators are clamped between the PCB and the case.  Secure the power supply to the case by screwing M3 bolts and nuts through the hole beneath the fuse -holder, the hole between L1 and L2 end through the 4 voltage regulators.

This will provide sufficient cooling for the + 5V and + 12V voltage regulators.  The -5 and - 12V voltage regulators have to be insulated from the earthed cooling surface with a mica or Teflon insulator and insulated washers in the mounting holes.

It is recommended that a small cooling fin is attached to the 33V zener diode (D3, TAA 550).

The PCB may be insulated on the primary side of the transformer (Caution mains voltage !) by applying a strip of insulating material (plastic or pertinax) with some adhesive tape against the solder side of the PCB.  Keep in mind “safety first”,  when experimenting with a power supply which has not yet been mounted permanently into the receiver case !

When the power supply has been completed check all voltages (+5, +12, +33, -5, -12V).

A simple PC interface for the receiver (refer fig. 8)

Please note that for normal use I recommend you use the ZETF circuit to set the regular frequencies.

This circuit uses a simple EPROM-based  I2C sequence generator.

The adjustment of the correct band and the VCO frequency is accomplished with a PC interface and a small software program.

The interface provides control signals using the I2C protocol and can also check whether the tuner has been adjusted correctly.

The interface was obtained from a Philips publication and is also suitable for controlling other I2C equipment,  such as audio processors,  satellite tuners,  car radios,  displays etc.

When using the interface,  be aware that it is not protected against high voltages or shorts and that the maximum cable length is 6 meters.

Use the same earthed power point for the PC,  the receiver,  the az/el rotor etc.,  or connect all cases together.  Should the voltage difference between e.g. the PC case and receiver case be too large the parallel port of the PC may be damaged !

Building the simple I2C PC interface

Drill the holes using a 0.7 - 0.85 mm drill.  Some holes need to be drilled from the other side.

The component lay-out is shown on the left of fig. 9   The PCB fits between the solder tabs of a 25 pin male D connector,  slide the PCB in position and ensure that the copper tracks line up exactly with all the solder tabs.

Use an IC socket with turned pin contacts for U1,  because pins 3, 7, 11 and 14 have to be soldered on the component side as well,  the resistors and the capacitor also have to be soldered on both sides wherever a pad is provided. The PCB has to be connected through on both sides of the print with a short length of wire between the 100nF capacitor and the D connector.

Solder some shielded cable (max. 6 meters) to the PCB to ensure the SDA and SCL signals are shielded.  The author used shielded stereo cable,  in which one shield wire was used to ground and the other shield is used for the + 5V supply.

The author uses a female 9 pin D connector on the other side of the cable,  as the receiver has been provided with a 9 pin male connector.  The pin connections are according to ELV (= a German electronics magazine)

P1,2,3	=	GND		> Shield left conductor

D9		P4,5,6	=	+ 5V		> Shield right conductor

P7	=	SCL		> Left conductor (white)

P8	= 	SDA		> Right conductor (red)

P9	=	No connection

Aligning the receiver

Use a properly fitting non-metal adjustment tool for aligning L4, L6 and L3.  Such a tool can be home made by sawing a piece of epoxy PCB material of 1.5 mm and filing the end to a taper.  The solid fibre glass material used in fishing rods is also suitable.

Aligning the VCO

An incoming signal is not required for this step.

Put the TUNE pot-meter P1 exactly in the center position.

Put S1 in the HRPT position (point Q in the VCO is connected to earth).  Now turn the slug of L3 until the frequency measured at junction L2/C11 is 36.7 MHz.

If this is not possible,  check and ensure that the internal capacitor has been removed and that the varicap voltage at junction R43/L1 is approx. 6V.  If still unsuccessful,  it is possible that the varicap diodes D1 and D2 are not of the correct value.  In this the case the value of C3 may be changed (increase if the frequency is too high,  decrease if the frequency is too low).

Be warned, a BB109 has a plastic rectangular housing and a white dot or white bar. Very often this type is mixed up with the BB109G that has a yellow bar. This BB109G varicap is not suited !

Now put S1 in the PDUS position (point Q not connected to earth) and adjust the frequency to 38.9 MHz with P5.  If this is not possible,  the value of varicap diode D8 is probably incorrect.  in this case the value of C61 maybe reduced if it is not possible to adjust the frequency high enough with P5.

Switch S1 several times from HRPT to PDUS and fine-tune L3 for HRPT and P5 for PDUS.

If a frequency counter is not available,  a scanner may be used,  tuned to the fourth harmonic;  4 X 36.7 = 146.8 MHz for HRPT and 4 X 38.9 = 155.6 MHz for PDUS.  Place the antenna wire approximately 10 cm above the VCO compartment.

By turning P1 it should be possible to tune the VCO 0.3 MHz higher and 0.3 MHz lower.

C43 has to be adjusted to ensure minimum distortion of the VCO signal,  for which purpose an oscilloscope with a minimum band-width of 30 MHz is required.  Place the probe on L2 on the C43 side.  If a 30 MHz oscilloscope is not available C43 should be set to the middle position (the rotor of the capacitor should be turned in half way).

In the remaining alignment procedure,  the tuner has to be adjusted (refer to the description of the control program below) and a Meteosat channel 2 signal should be available.

A test signal on 137.5 MHz could also be used,  however since this signal does not contain any data (modulation),  P3 cannot be adjusted and the eye pattern will not be visible.

Alignment of the PDUS IF

Switch the receiver to PDUS and adjust the RF gain knob to maximum and the TUNE knob to the center position.  Look for a PDUS signal on channel 2 (minimum 1.2 m dish with a good LNA is essential).

Connect the oscilloscope probe (min. 30 MHz) to the test point next to L8 and adjust L4 for maximum amplitude.  Do the same for C48, L6 and C39.  Repeat this procedure and see whether small changes in the position of L4, C48 and L6 produce a cleaner and straighter sine wave on the oscilloscope,  ensuring that the amplitude remains approximately constant.  Turn the RF gain down until the signal becomes noisy and repeat the procedure.

If L4, C48 and L6 are adjusted too far,  the tops of the sine wave will “tilt” (distortion).

Alignment of  HRPT IF

Switch the receiver to HRPT and turn the RF GAIN knob to full and the TUNE knob to the center position.  Set the tuner to receive an HRPT signal on e.g. 1698 MHz.,  it is not necessary for a signal to be present.

Connect the oscilloscope probes to the test point next to L 12.  and adjust C23 for maximum amplitude of noise or signal (if this is present).  Later when an HRPT signal is being received,  C23 may be fine-tuned.  Keep an eye on the signal strength meter during this process.

If an oscilloscope is available which is capable of displaying 36.7 (HRPT),  or 38.9 MHz (PDUS),  observe the signal strength meter and connect the oscilloscope probe to the test point following the mixer and make adjustments with the aim of producing minimum distortion of the eye pattern.

Alignment of the signal strength meter

Maximum meter deflection is adjusted with P7 and response sensitivity ( the point at which the meter moves out of the 0 region),  is adjusted with P6.

Switch the receiver to PDUS and tune to Meteosat channel 1 or 2 with the RF GAIN control adjusted to full.  Adjust P7 for maximum deflection and switch to HRPT (you should now receive noise).  Adjust P6 to the point where the meter just starts to move away from zero.  Because P6 and P7 mutually affect one another these steps have to be repeated a few times.  It is possible that full deflection cannot be obtained with P7,  as this is largely dependent on the gain of the aerial,  Meteosat converter and LNA.

Adjustment of the TUNE meter

The TUNE meter works only when the receiver is locked to an incoming signal

Whilst receiving a signal (may also be Meteosat channel 1 or a test signal),  turn the TUNE control,  the TUNE meter should deflect,  when continuing to turn the pot,  the meter should go back,  showing that the VCO is no longer locked.  Adjust P4 for maximum deflection (but not against the needle stop),  just before the receiver looses lock.  Should the TUNE meter not be sensitive enough,  increase the value of R55 (depending on meter 100K to 470K).

Should the TUNE meter not be positioned at mid scale when there is no incoming signal,  U4 (TL084) will have to be replaced.

If the TUNE knob is not positioned in the middle,  following adjustment,  the VCO frequency will have to be adjusted slightly (L3 for HRPT and P5 for PDUS).

Checking AGC

Pin 5 on those MC 1350 IC’s which are activated should indicate approx. 5.6 to 5.9 volt (DC).

Adjustment of P3 (only possible when a digital signal is being received)

Connect oscilloscope probe to pin 1 of U2 (166 or 666 KHz) and turn P3 for maximum amplitude without signal distortion.

As an indication,  a number of measured values during PDUS reception are stated below:

Measuring position			signal

U7,  pin1				38.9 MHz sine wave approx. 90 mV p/p

U8,  pin4				38.9 MHz sine wave 120 mV peak/peak

U8,  Pin1				38.9 MHz sine wave 0.8 V peak/peak

Test point near L8			38.9 MHz sine wave 0.3 V peak/peak

Test point near mixer			166 KHz “eye-pattern” approx. 0.2 V p/p

U2,  pin1				166 KHz “eye-pattern” approx. 0.4 V p/p

U1,  pin3				38.9 MHz sine wave approx. 0.3 V p/p

L2/C11					38.9 MHz sine wave approx. 0.15 V p/p

U7 + U8,  pin 5				5.6 to 5.8 V DC

Similar signal levels should be obtained during HRPT reception,  however the IF frequency will be 36.7 MHz and the eye pattern is 666 KHz.  U9 and U10 should be measured instead of U7 and U8.

The control program for the tuner

In order to tune the correct frequency of the UV816 tuner a PC parallel port is required.  Ensure that both the PC and the receiver share the same earth point !

There are two programs to control the tuner.

In order to use the program written in basic “zetf.bas”, QBASIC.EXE is required (standard DOS V5 and higher).

The Philips “IICTV” program may also be used for test purposes.  This is however it a bit more complicated because of the many control options.

After start up,  the zetf.bas program asks the following:

PDUS of HRPT ontvangst (reception) P/H ?	key in P and press <Enter>

PDUS ontvangst gekozen (reception selected), MF (IF) = 38.9 MHz.

Geef frequentie op (Enter frequency) (134, 137.5, 141, 150 MHz) Input 137.5 press <Enter>

Ingevoerd (Entered) 137.5

Is dit juist ? (j/n)  (Is this correct ? (y/n)  Input j,  followed by <Enter>

The program will now show the entered values and will indicate that the frequency for the receiver has been adjusted by showing the sentence “PLL is gelocked” (“PLL is locked”).

The program may be terminated by entering <Alt> F,  followed by X.  The frequency will remain as entered,  as long as the receiver remains turned on.  The PC and parallel port may now be used for other programs.  If desirable a printer switch can be used to allow quick switching over to e.g. the HRPT decoder of Rob Alblas.

If the PC interface does not function correctly,  the receiver is not connected or the tuner is faulty,  the program will return an error message,  together with an indication of possible causes.

Which receiving frequencies to enter

LNC 1700 Meteosat converter			137.5 for PDUS reception on 1694.5 MHz

switched to channel 2				141.0 for HRPT reception on 1698 MHz

150.0 for HRPT reception on 1707 MHz

LNC 1700 Meteosat converter			141.0 for PDUS reception on 1694.5 MHz

switched to channel 1				144.5 for HRPT reception on 1698. MHz

150.5 for HRPT reception on 1707 MHz

Please note that the use of channel 2 on the Meteosat converter is to be preferred,  since HRPT reception using channel 1 may suffer from amateur radio interference in the 144-146 MHz band.

Operating instructions for the receiver

Switch the HRPT/PDUS switch to the desired position and adjust the RF GAIN to full,  with the TUNE control set to the mid position.  Boot the PC and start the zetf.bas program and enter the required frequency.  Once completed exit the program by keying in <Alt> F-X.

When there is an incoming signal,  the signal meter will indicate this.  Even when there is hardly any movement in the signal meter,  the TUNE meter will show some movement by turning the TUNE knob.

When receiving PDUS no further action is needed,  once “tuning” is completed (TUNE meter in mid position).

Reception of HRPT,  using manual aerial positioning will require turning the RF GAIN knob back if the signal meter indicates an excessively high reading.

Check the TUNE meter regularly and if the needle varies considerably from the mid position this can be corrected by adjusting the TUNE knob.

Although the AFC system will adjust the VCO without regular adjustment of the TUNE knob,  the “untuned VCO may not lock quickly if the signal is lost suddenly (due to terrain obstructions between satellite and dish or tracking errors).

This covers the description of the HPR137.  Work is in progress to expand the receiver,  develop an aerial and LNA.  Best wishes for a successful completion of the HPR137 receiver !

-oOo-

Addendum receiver circuit

A small error has crept into the receiver circuit published in part 1 (De Kunstmaan April 1997);  Switch S1 must connect +12 V through to pin 1 of relay TQ2 when in the PDUS mode (and not in the HRPT position as is drawn).  This has already been corrected in the connection diagram in this part (fig. 9).

Additional information for part 1

During construction of several power supplies it has been observed that when using a light duty (14-18 VA) transformer,  the secondary 12 V winding drops excessively under full load (less than 15 V on C1).  Under these conditions the LM7812 voltage regulator will deliver less than 12 V and the voltage is not stabilized.  This can be corrected by using a “low drop” voltage regulator such as the L4812.

Of course it is also possible to use a transformer with a higher secondary voltage,  13-14 V would be ideal,  however the next available value is 15 V.

If a transformer with a rating of 12 V 24 VA is used,  the voltage drop is less and the LM 7812 can be used.

If a Meteosat converter, 1700 MHz LNA and other options are to be supplied,  a 18 VA rated transformer must be used.

The PCB’s for the receiver,  power supply and PC interface are available from Printservice Hans Beukinga.

Arne van Belle with thanks to Jaap van Rotten.

Appendix;  Parts list for the receiver

Bibliography

ESA; Prototype PDUS system,  University of Dundee

Weathersat Ink a/o. Fall 1992; The HRPT receiver

Various RIG articles related to HRPT and PDUS

UKW Berichte 4/95; Ein NOAA Empfaenger

CQ-DL 5/93; PLL Synthesizer-Empfaenger fuer 47 bis 860 MHz.

Elektuur 9/93; VHF/UHF Kabeltuner

Philips components;			The I2C-bus Specification

User’s Guide to I2C-bus Control Programs

Datasheet TSA 5510 and UV 816 series

The Internet and many other sources.

Future expansion for the receiver

-	A home-built LNA suitable for SDUS, PDUS and HRPT

-	A directional Helical aerial for HRPT reception

-	The use of old antenna rotors for use with light weight HRPT aerials

-	Expansion to allow APT and FM reception for the HPR137 (47 to 860 MHz.)

Parts list for the HPR137

Resistors

10E		4	R22 R23 R26 R27

33E		1	R6

47E		2	R50 R51

51E		1	R42

100E		10	R11 R16 R24 R25 R28 R30 R31 R33 R49 R56

180E		2	R7 R8

270E		1	R4

330E		1	R32

470E		1	R14

560E		1	R13

680E		1	R12

1K			11	R1 R10 R15 R17 R18 R34 R36 R37 R38 R43 R53

1K2		1	R35

2K7		1	R20

3K3		2	R9 R40

4K7		2	R5 R21

5K6		1	R2

8K2		1	R52

10K		5	R3 R19 R41 R44 R45

15K		1	R29

22K		1	R39

33K		1	R54

47K		1	R47

100K		2	R46 R48

470K		1	R55

Potentiometers:

1K			1	P4	PT10 Horizontal mount  (PIHER)

2K5		1	P6	PT10 Horizontal mount  (PIHER)

5K			1	P5	Multi Turn Trimmer 10*10*5 mm Top adjusting (BOURNS, SPECTROL)

10K		1	P3	PT10 Horizontal mount  (PIHER)

470K		1	P7	PT10 Horizontal mount  (PIHER)

10K Lin.	1	P2	Single Turn with shaft 4/6MM

1K	  Lin.	1	P1	SPINDLE  4/6MM

Capacitors:

2..10PF		2	C23 C39	Polyethylene Film Trimmer 7.5MM

2..15PF		1	C48        	Polyethylene Film Trimmer 7.5MM (or 2..22PF)

3..30PF		1	C43        	Polyethylene Film Trimmer 7.5MM

4n7			1	C59

10PF		2	C3 C17        	NP0

15PF		1	C61        	NP0

33PF		1	C11        	NP0

82PF		1	C51        	NP0

100PF		1	C12        	NP0

470PF		2	C63 C64

1NF		1	C50

2N2		2	C27 C28

3N9		1	C10

10NF		6	C2 C8  C9  C33 C34  C62

22NF		15	C1 C15 C18 C20 C22 C24 C30 C32 C35 C40 C41 C42                     						C44 C45 C57

68NF		1	C7	MKM 5mm

100NF		14	C4 C6 C14 C16 C26 C38 C46 C47 C53 C55 C56                                    						C58 C59 C60

470NF		1	C13

All C's with exception of C7 have pitch 2.54 mm

3U3		1	C25			TANTALUM

10UF		5	C19 C21 C29 C31 C52	TANTALUM

22UF		1	C49			TANTALUM

100UF		4	C5 C36 C37 C54        	ELECTROLYTIC 16V

Inductors:

0.47UH		1	L2		Choke axial

100UH		1	L1		Choke axial

2U2		1	L15	Choke axial

12UH		7	L5 L7 L9 L10 L11 L13 L14  5 turns. 0.3mm enameled Cu on ferrite bead 3*4mm

K3267		3	L3 L4 L6  TOKO 10mm KXCSK3267, remove internal C for L3 !

TORROID	2	L8	L12

Torroidal ring core FT23-61 primary 5+5 turns, secondary 2.5 turns 0.3mm enameled Cu

Semi-conductors:

1N4148		3	D5 D6 D7

AA119		2	D3 D4

BB109		3	D1 D2 D8  VHF varicap 25..34PF (or BB909A, BB114)

BF245		1	Q1

LM311		1	U5

7404		1	U6 (do not use 74LS04 or 74HC(T)04)

MC1350	5	U2 U7 U8 U9 U10   IF amplifier with gain-control

MC1648	1	U1   VHF VCO with AGC (Motorola)

TL084		2	U3 U4

Others:

UV816PLL		1	Hyper tuner with Synthesizer  Note: see additional note for equivalent tuner types

OFW			1	OFWG 3250 SAW-filter (don't use Stereo or Nicam types, these are to wide)

TQ2			1	RE1	12V Mini HF relay  (NAIS)

TIN				1	HF-tin plate box 160*100*30 mm standard Euro-dimension

Feed-thru C		12	Ceramic feed-through C 470..1500PF

SBL-1			1	M1	Double balanced mixer, also usable SCM-1 (smd) or other +7dBm mixer

S-meter		1	100..500 UA moving coil panel meter (e.g. VU-meter)

Tune-meter		1	100..500 UA moving coil panel meter with center zero (VU-meter can be modified)

S1				1	Switch 2x pole

1	Right Angle TVAnt. Plug for UV816

Parts list for universal power supply:

Resistors:

100E			1	R2

120E			1	R1

Capacitors:

10UF 63V		1	C5		Electrolytic upright mounting

100NF			1	C6				MKM 5mm

220NF			6	C7 C8 C9 C10 C11 C12	MKM 5mm

220UF 63V		1	C4		Electrolytic upright mounting

1000UF25V		1	C2		Electrolytic upright mounting, height max. 33mm

4700UF25V		1	C1		Electrolytic upright mounting, height max. 33mm

Semiconductors:

LM7912		1	VR2	1A.voltage regulator -12V TO-220

LM7905		1	VR4	1A voltage regulator -5V TO-220 (optional, not required for							HPR137)

L4812			1	VR1	1A voltage regulator 12V, Low Drop TO-220

When transformer above 24 VA or transformer with 13-	15V LM7812 can be used for  VR1

LM7805		1	VR3	1A voltage regulator 5V TO-220

1N4001			2	D1 D2

TAA550		1	D3		33V Zener metal-can

B40C2200		1	B1		Bridge rectifier, minimum 40V 2.2A

BF256B		1	Q1

Others:

TRANSFORMER	 1	TR1	PCB transformer 2x115V 2x12V minimum14VA (e.g. ERA)

Note:						TR1 must have minimum rating of 20VA when the Meteosat converter, 1700MHz LNA and other options have to be supplied by the receiver.

FUSE 	HOLDER	F1		PCB-fuse-holder surface mount 22mm + cover

FUSE				1			F250-500mA 5x20mm, depending on transformer used

PRINTKR.2			1	J1		PCB connector block 2 pole, pitch 5.08mm

PRINTH.2x3		4    J2 J3 J4 J6	PCB-header 2x3, pitch 2.54mm

PRINTH.2x4		1    J5		PCB-header 2x4, pitch 2.54mm

Choke				4	L1 L2 L3 L4	"Pig’s nose" with 2.5 turns

TO-220 Isolation	2	isolate VR2 en VR4 from cooling surface with mica

foil and plastic insulator

1	push-on cooling fin for D3 (TAA550, TO-18 case)

If a PCB type transformer is not used, a 3 pole connector should be mounted on the PCB (next to the rectifier B1),  and the 2 pole connector and fuse holder are not required.

Parts list simple PC interface for receiver:

Resistors:

10K			6	R1 R2 R3 R4 R5 R6

Capacitors:

100NF			1	C1	surface mount 2.54 mm

Semi-conductors:

74LS05			1	U1

1	IC socket14P with turned pin contacts (because of double sided soldering)

Others:

DB25 MALE	1	D-connector 25 pin male + cover (with a little filing this will fit on the

PCB)

DB9 FEMALE	1	D-connector 9 pin female + cover shielded (twin) stereo-cable approx. 2 meter