Frequency table for ZETF control

| Switch position on ZETF | HRPT/PDUS Mode switch | Tuner receives | Downconverter receives | Used by Sat |
| --- | --- | --- | --- | --- |
| 1 | HRPT | 141.0 MHz | 1698.0 MHz | NOAA 10+12 |
| 2 | HRPT | 143.5 MHz | 1700.5 MHz | Feng-Yung 1C 1 |
| 3 | HRPT | 145.5 MHz | 1702.5 MHz | Seatstar 2 ,NOAA15 4 |
| 4 | HRPT | 150.0 MHz | 1707.0 MHz | NOAA 9+11+14 |
| 5 | PDUS | 134.0 MHz | 1691.0 MHz | MET5 (INDOEX) 3 |
| 6 | PDUS | 137.5 MHz | 1694.5 MHz | MET7 |

Notes:

All frequencies mentioned when using a downconverter that converts 1694.5 MHz to 137.5 MHz !

1 	Feng-Yung 1C transmits "CHRPT", unencrypted, but modifications to receiver and decoder may be necessary.

2	Seastar (ORBCOMM) is encrypted now.

3	For direct reception of MET-5 PDUS, you need a free view on the East horizon (from the Netherlands) !

4	1702.5 MHz is the backup channel for NOAA15 HRPT now the 1698 MHz transmitter broke down.

Using other tuners in the HPR137 receiver:

The UV816 tuner is not produced anymore, but Philips produces compatible tuners.

Look for old TV sets from Philips and you will often find one of the tuners stated below.

These type markings are printed in dot matrix style on the topside of the box (opposite the connection-pins)

The UV916E/IEC is used as replacement for the UV816 and can be ordered in most Philips repair centres.

Barend Hendriksen now sells the UV916S, almost the same as the UV916E/IEC/ but with different I2C address.

| Tuner type | UV816 UV916E/IEC | FQ816ME a FQ916ME a | UV916S | UV1216 b UV1316 b |
| --- | --- | --- | --- | --- |
| I2C Address | 194 | 194 | 192 | 192 |
| I2C Control-Byte | 142 | 142 | 142 | 140 (50 KHz step) |
| I2C Lowband Select | 96 | 224 | 96 | 160 |
| ZETF File for EPROM (Standard 4MHz Xtal in tuner) | Zetf6.hex | Zetf6.hex | Zetf916s.hex | Not available yet |
| Link from EPROM pin1-pin2 C | Closed | Open | Closed | - |
| QBASIC PC Control program | Zetuvfq.bas | Zetf.bas | Zetf916.bas | Zetf1216.bas |
| Maximum AGC voltage | +12V | +12V | +5V | +5V |

Notes:

a 	These Tuners consist of a tuner section and an attached IF filter+demodulator section, use a small metal hacksaw to remove this section.

b	Please note that the UV1216 and UV1316 tuners are not pin-compatible, they don’t fit directly to the HPR137 PCB. The UV1316 has not been tested yet.

C	On the ZETF PCB is a track running from EPROM pin1 to pin2, remove track to open, normally closed.

EPROM files for tuners with 3.2 MHz Xtal (giving 50 Khz stepsize, standard = 4.0 MHz, 62.5 Khz stepsize) can be supplied on request.

Modification for the UV916S:

The UV916S uses a Motorola synthesiser-chip instead of the commonly uses TSA5510 to TSA5514 range. This synthesiser does not support in-lock-detection, i.e. you can set frequency using I2C commands, but there are no commands to confirm if VCO is actually locked on this frequency. The ZETF frequency controller doesn’t use this feature, the PC control Qbasic-program zetf916s.bas has been adapted for this tuner.

Modification for UV916S, UV1216, UV1316:

These tuners don’t need a+12V supply anymore, all electronics work on the +5V supply.

Therefore  they don’t have a +12V connector pin.

As a result of this, the AGC voltage from the GAIN-pot, must be lowered.

Connect the most right pin (see componentlayout, pin “R”) from the GAIN-pot to +5V instead of +12V !

Looking for other type or brand tuners:

Other manufacturers also use so-called hyperband tuners. For example Grundig has a complete range and Sony uses the UV916.

If you find another tuner, try to open the tuner and look for a I2C controlled Synthesiser/bandswitch IC like the TSA5510 to 5514 or the SDA3202. Close to this IC there should be a 4.0 MHz Xtal.

There generally is also a Mixer/VCO IC like the TDA5330 or TDA5331 present.

Both IC's can be normal or SMD-type mounted on the solder-side.

All tuners mentioned above have three bands, Low band is 47 to 170 MHz. Mid band is 170 to 450 MHz, High band is 450 to 860 MHz. We only use the Low band.

If you find a tuner without synthesiser, this could also be used with certain restrictions.

For example older tuners like UV816-256 or UV615/616 have been used successfully before.

Omit the PC interface, ZETF control PCB and mount a six-way rotary switch instead.

Use multi-turn potmeters to set tuning voltage for every switch position. Connect all pots at one side to + 33V, the other sides all to ground and every wiper to a switch contact.

Instead of connecting the main PCB to +33V, connect the common contact from the rotary switch.

Note that frequency drift may occur when the tuner warms up or when tuning voltage is not stable.

Using the TUNE pot can compensate limited frequency drift.

Modifications for CHRPT (Feng-Yung 1C):

CHRPT is a “boosted” HRPT-standard (Chinese HRPT).

It utilises 10 spectral channel instead of the normally used 5 channels (Note: Seastar transmits 8 spectral channels, but at lower resolution).

While image resolution from CHRPT is equal to HRPT, this results in a data rate that is twice as high !

The CHRPT data-rate is 2*665 KHz = 1330 KHz.

To receive and demodulate this signal, the receiver should have an IF-band-width

of approx. 4*1330=5320 KHz.

The SAW filter used in the HPR137 can cope with this bandwidth, so basically the receiver does not need modifications for CHRPT.

But the first test showed that signal strength from Feng-Yung 1C is a lot weaker than NOAA signal.

Wil Pfeifer Uses a 1 meter Dish an d good LNA, he can decode CHRPT using the HPR137 and Rob Alblas decoder.

But you must have a decoder that can handle the CHRPT data rate !!!